SL - Extended Controller API

STUDIOLOGIC ® 2025

Introduction

The SL - Extended Controller API provides comprehensive functionality for managing the connection, communication, and display interaction between SL-MKII and the remote device, ensuring optimal performance and user experience. The communication protocol employed is MIDI, leveraging the transmission and reception of 'System Exclusive' messages. This approach ensures compatibility and flexibility in data exchange between devices, adhering to MIDI standards while accommodating proprietary functionalities specific to the SL-MKII and the connected remote device.

What are Sysex Messages?

MIDI (Musical Instrument Digital Interface) System Exclusive messages, commonly abbreviated as Sysex, are MIDI messages designed to transmit specific information between MIDI devices from the same manufacturer or model.

Sysex messages are a special category of MIDI messages that allow the transmission of proprietary data between devices. These messages are not standardized, so their interpretation varies depending on the manufacturer and model of the device.

Structure of Sysex Messages

Sysex messages consist of a series of bytes that start with a specific status byte (0xF0) and end with an end-of-message byte (0xF7). The general structure is as follows:

- Start status byte: 0xF0 (240 in decimal).

- Manufacturer identifier: One or three bytes that identify the manufacturer of the device. For example, 0x41 for Roland, 0x7E and 0x7F for universal messages. The FATAR (Studiologic) identifier is 0x00 (byte 0), 0x20 (byte 1) and 0x1A (byte 2)

- Specific data: A sequence of bytes containing manufacturer-specific data. These bytes vary depending on the manufacturer and the type of message.

- End-of-message byte: 0xF7 (247 in decimal).

Example of a Sysex Message

An example of a Sysex message to modify a specific parameter on a synthesizer might look like this: F0 [00 20 1A] [0E 01] [04 01 00] F7

- F0: Start of the Sysex message.

- 00, 20 and 1A: Manufacturer identifier (FATAR).

- 0E: Product ID (i.e. SL-MKII ID).

- 01: Host ID (e.g. NumaPlayer, Camelot etc.)

- 04 01 00: Specific data of the message.

- F7: End of the Sysex message.

SYSTEM MESSAGE

The System Message establishes the connection and disconnection policy between the SL-MKII and the remote device. This message serves a dual purpose and can be both sent and received.

Message Types:

- System: Establishes connection and disconnection policies between SL-MKII and the remote device.

- Bitmap: Allows for the rendering of bitmap images on the SL-MKII display.

- Rectangle: Enables the drawing of geometric shapes, such as rectangles, on the SL-MKII screen.

- Text: Facilitates the display of textual information on the SL-MKII LCD.

- Clear: Clears the SL-MKII display, providing a clean canvas for new content.

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x00 →  item type: system message

- 0xFC→ function: SEE system_type enumerator

- 0xtt → 1-st string character  (only for function 0, or better, login request)

...→ ““

- 0xtt → n-th string character

- 0x00 → string terminator

- 0xF7 → stop sysex

The byte labeled 0xFC sets the type of request, SEE system_type enumerator.

typedef enum

{

SYSTEM_LOGIN_REQUEST = 0,

SYSTEM_LOGIN_CONFIRM,

SYSTEM_LOGOUT_REQUEST,

SYSTEM_LOGOUT_CONFIRM,

SYSTEM_STANDBY,

SYSTEM_RESTART

} system_type;

0 → SYSTEM_LOGIN_REQUEST,

1 → SYSTEM_LOGIN_CONFIRM,

2 → SYSTEM_LOGOUT_REQUEST,

3 → SYSTEM_LOGOUT_CONFIRM,

4 → SYSTEM_STANDBY (app is connected but can’t drive SL),

5 → SYSTEM_RESTART (app can restart its activity but first it must do a complete

display refresh)

When sending the value 0x00, which is the 'Login Request' (i.e., SYSTEM_LOGIN_REQUEST), the remote device requests SL-MKII to appear in the list of 'AVAILABLE APPS'. This is accessible through the 'APP' button.

Note: This message needs to be sent continuously, for example, every 1 or 2 seconds. If the message is no longer received by SL-MKII after a certain time (10 seconds), the APP is removed from the list. The message must contain a text string 0xtt that identifies the device sending it (APP).

You can request to be removed from the list by sending a 0x02 (SYSTEM_LOGOUT_REQUEST) logout request message.

These concepts will be further explored in the examples.

It's important to understand that when sending a login request, it simply makes the APP appear on the list. When SL-MKII confirms the login of the APP from the list, SL-MKII sends the value 0x01 (login confirmation). Upon receiving this value, the APP should proceed to refresh the display with its contents. Similarly, when receiving 0x02 (logout request), the APP should suspend sending messages but first send a 0x04 (logout confirmation).

System Message Details:

- Login Request (0): Initiates a request for the remote device to appear in the list of 'AVAILABLE APPS' on SL-MKII.

- Login Confirmation (1): Confirms the successful login of the remote device, allowing it to proceed with display refresh and content rendering.

- Logout Request (2): Requests removal from the 'AVAILABLE APPS' list on SL-MKII.

- Logout Confirmation (3): Confirms the successful logout from the 'AVAILABLE APPS' list on SL-MKII.

- Standby Notification (4): Notifies that the app is connected but cannot control SL-MKII at the moment.

- Restart Notification (5): Indicates that the app can restart its activity but must perform a complete display refresh first.

Usage Guidelines:

- Continuous Transmission: It is recommended to continuously transmit the login request message (0) to ensure the presence of the app in the list of 'AVAILABLE APPS'. Failure to receive this message may result in the removal of the app from the list.

- Identification String: Each message should contain an identification string (0xtt) to uniquely identify the sending device (APP).

- Logout Procedure: When receiving a logout request (2), the app should suspend message transmission and promptly send a logout confirmation (3) before ceasing activity.

- Login Confirmation Handling: Upon receiving a login confirmation (1) from SL-MKII, the app should proceed with display content refresh to synchronize with SL-MKII.

Messages coordinates

Before proceeding with a detailed explanation of each, it is necessary to clarify the functioning of the X and Y coordinate mechanism.

Each byte of the sysex message cannot express values greater than 127. This is because MIDI uses 7 bits for each byte. The SL 88 has a display size of 320 x 240 pixels, so a single byte is not enough to correctly express the X and Y positions. To achieve this, two bytes are sent (Most Significant Byte and Least Significant Byte). This ensures that the data can cover the entire display size.

The code below can be integrated to convert the 16-bit value for the X or Y axes. The return value is a structure (tuple) with two fields for the MSB and LSB values. This principle will become clearer when the examples are presented.

//------------------------------------------

struct MsbLsb

{

MsbLsb(uint8_t m, uint8_t l) : msb(m), lsb(l)

{

}

uint8_t msb = 0;

uint8_t lsb = 0;

};

//------------------------------------------

MsbLsb SysexManagerNew::get7BitMsbLsb(uint16_t val) {

uint8_t msb = val >> 0x07;

uint8_t lsb = val & 0x7F;

return MsbLsb(msb, lsb);

}

PLOT LCD BITMAP

This message accesses a bitmap from the SL-MKII memory and draws it at the X and Y position, expressed in pixels.

Byte Breakdown

- 0xF0: Start SysEx

- 0x00: Manufacturer ID byte 0

- 0x20: Manufacturer ID byte 1

- 0x1A: Manufacturer ID byte 2

- 0x0E: Product ID

- 0xID: Host ID (app identification code)

- 0x04: Item type (display LCD)

- 0x03: Function (3 = draw bitmap)

- 0xXX_MSB: MSB of the x-coordinate of the top left vertex of the bitmap

- 0xXX_LSB: LSB of the x-coordinate of the top left vertex of the bitmap

- 0xYY_MSB: MSB of the y-coordinate of the top left vertex of the bitmap

- 0xYY_LSB: LSB of the y-coordinate of the top left vertex of the bitmap

- 0xII_MSB: MSB of Bitmap number, SEE list_icon enumerator

- 0xII_LSB: LSB of Bitmap number, SEE list_icon enumerator

- 0xJJ: Bitmap version or 0x7F if there is only one single version

- 0xF7: Stop SysEx

The message that distinguishes this function is:

- 0x03: Function (3 = draw bitmap)

By sending the value 3, you request the SL-MKII firmware to access a bitmap in memory and draw it at the X and Y coordinates. After specifying the coordinates (see above for clarifications), it will be necessary to provide an index that refers to the available images. See Bitmap List to know the available images.

The last byte to be sent, valid only for KNOB family, even if it is necessary to send it to make the message consistent, will refer only to some types of images, for example, the KNOB family (see the list above).

- 0xJJ: Bitmap version or 0x7F if there is only one single version

The rotation of each KNOB is implemented through a strip-film of 12 images. The "Bitmap version" sets one of these 12 steps.

Bitmap List

Below is the list of available bitmaps with their corresponding bitmap numbers:

typedef enum

{

// JOYSTICK

IMG_JT_LEFT_BLACK,

IMG_JT_LEFT_GREY,

IMG_JT_LEFT_ZONE1,

IMG_JT_LEFT_ZONE2,

IMG_JT_LEFT_ZOME3,

IMG_JT_LEFT_ZONE4,

IMG_JT_LEFT_AUDIO1,

IMG_JT_LEFT_AUDIO2,

IMG_JT_LEFT_AUDIO3,

IMG_JT_LEFT_AUDIO4,

IMG_JT_LEFTRIGHT_BLACK,

IMG_JT_LEFTRIGHT_GREY,

IMG_JT_LEFTRIGHT_ZONE1,

IMG_JT_LEFTRIGHT_ZONE2,

IMG_JT_LEFTRIGHT_ZONE3,

IMG_JT_LEFTRIGHT_ZONE4,

IMG_JT_LEFTRIGHT_AUDIO1,

IMG_JT_LEFTRIGHT_AUDIO2,

IMG_JT_LEFTRIGHT_AUDIO3,

IMG_JT_LEFTRIGHT_AUDIO4,

IMG_JT_RIGHT_BLACK,

IMG_JT_RIGHT_GREY,

IMG_JT_RIGHT_ZONE1,

IMG_JT_RIGHT_ZONE2,

IMG_JT_RIGHT_ZONE3,

IMG_JT_RIGHT_ZONE4,

IMG_JT_RIGHT_AUDIO1,

IMG_JT_RIGHT_AUDIO2,

IMG_JT_RIGHT_AUDIO3,

IMG_JT_RIGHT_AUDIO4,

IMG_JT_UPDOWN_BLACK,

IMG_JT_UPDOWN_WHITE,

IMG_JT_LEFTRIGHT_WHITE,

// Rotate

IMG_JT_ROTATE_BLACK,

IMG_JT_ROTATE_WHITE,

IMG_JT_ROTATE_GREY,

IMG_JT_ROTATE_GREY_MUTE,

IMG_JT_ROTATE_ZONE1,

IMG_JT_ROTATE_ZONE2,

IMG_JT_ROTATE_ZONE3,

IMG_JT_ROTATE_ZONE4,

IMG_JT_ROTATE_AUDIO1,

IMG_JT_ROTATE_AUDIO2,

IMG_JT_ROTATE_AUDIO3,

IMG_JT_ROTATE_AUDIO4,

IMG_JT_SELECT_BLACK,

IMG_JT_SELECT_WHITE,

IMG_JT_SELECT_BLACK_SMALL,

IMG_JT_SELECT_WHITE_SMALL,

IMG_JT_SELECT_GREY_BLACK_SMALL,

IMG_JT_SELECT_GREY_WHITE_SMALL,

// APPLY

IMG_STORE_OK_WHITE,

IMG_STORE_OK_BLACK,

IMG_STORE_OK_GREY,

IMG_STORE_OK_GREY_MUTE,

IMG_STORE_OK_ZONE1,

IMG_STORE_OK_ZONE2,

IMG_STORE_OK_ZONE3,

IMG_STORE_OK_ZONE4,

IMG_STORE_OK_AUDIO1,

IMG_STORE_OK_AUDIO2,

IMG_STORE_OK_AUDIO3,

IMG_STORE_OK_AUDIO4,

IMG_STORE_CLOSE_GREY,

// KNOB

IMG_KNOB_WHITE_CENTER,

IMG_KNOB_WHITE,

IMG_KNOB_GREY,

IMG_KNOB_ZONE1,

IMG_KNOB_ZONE2,

IMG_KNOB_ZONE3,

IMG_KNOB_ZONE4,

IMG_KNOB_AUDIO_1,

IMG_KNOB_AUDIO_2,

IMG_KNOB_AUDIO_3,

IMG_KNOB_AUDIO_4,

// ARROW LEFT

IMG_ARROW_LEFT_WHITE,

IMG_ARROW_LEFT_BLACK,

IMG_ARROW_LEFT_GREY,

IMG_ARROW_LEFT_GREY_MUTE,

IMG_ARROW_LEFT_ZONE1,

IMG_ARROW_LEFT_ZONE2,

IMG_ARROW_LEFT_ZONE3,

IMG_ARROW_LEFT_ZONE4,

IMG_ARROW_LEFT_AUDIO1,

IMG_ARROW_LEFT_AUDIO2,

IMG_ARROW_LEFT_AUDIO3,

IMG_ARROW_LEFT_AUDIO4,

// ARROW RIGHT

IMG_ARROW_RIGHT_WHITE,

IMG_ARROW_RIGHT_BLACK,

IMG_ARROW_RIGHT_GREY,

IMG_ARROW_RIGHT_GREY_MUTE,

IMG_ARROW_RIGHT_ZONE1,

IMG_ARROW_RIGHT_ZONE2,

IMG_ARROW_RIGHT_ZONE3,

IMG_ARROW_RIGHT_ZONE4,

IMG_ARROW_RIGHT_AUDIO1,

IMG_ARROW_RIGHT_AUDIO2,

IMG_ARROW_RIGHT_AUDIO3,

IMG_ARROW_RIGHT_AUDIO4,

// ARROW UP

IMG_ARROW_UP_WHITE,

IMG_ARROW_UP_BLACK,

IMG_ARROW_UP_GREYED,

// ARROW DOWN

IMG_ARROW_DOWN_WHITE,

IMG_ARROW_DOWN_BLACK,

IMG_ARROW_DOWN_GREYED,

// SPEAKER

IMG_SPEAKER_GREY_MUTED,

IMG_SPEAKER_ZONE1,

IMG_SPEAKER_ZONE2,

IMG_SPEAKER_ZONE3,

IMG_SPEAKER_ZONE4,

IMG_SPEAKER_AUDIO1,

IMG_SPEAKER_AUDIO2,

IMG_SPEAKER_AUDIO3,

IMG_SPEAKER_AUDIO4,

// ARROW RANGE

IMG_RANGE_GREY_MUTE,

IMG_RANGE_GREY,

IMG_RANGE_WHITE,

IMG_RANGE_ZONE1,

IMG_RANGE_ZONE2,

IMG_RANGE_ZONE3,

IMG_RANGE_ZONE4,

// KEYBOARD

IMG_KEYBOARD_GREY_MUTE,

IMG_KEYBOARD_GREY,

IMG_KEYBOARD_WHITE,

IMG_KEYBOARD_ZONE1,

IMG_KEYBOARD_ZONE2,

IMG_KEYBOARD_ZONE3,

IMG_KEYBOARD_ZONE4,

// DIALOG

IMG_BACK_GREY,

IMG_ENTER_GREY,

IMG_EDIT_GREY,

// TOGGLE

IMG_TOGGLE_OFF,

IMG_TOGGLE_ON,

// STUDIOLOGIC ICON

IMG_SL_ICON

} list_icon;

Bitmap Family Sizes

Here are the formatted dimensions of the images for better readability:

JOYSTICK ICONS

- Width: 20 pixels

- Height: 20 pixels

APPLY ICONS

- Width: 20 pixels

- Height: 20 pixels

KNOB ICONS

- Width: 61 pixels

- Height: 54 pixels

ARROW ICONS

- Width for up/down arrow: 16 pixels

- Height for up/down arrow: 8 pixels

- Width for left/right arrow: 8 pixels

- Height for left/right arrow: 16 pixels

SPEAKER ICONS

- Width: 16 pixels

- Height: 12 pixels

RANGE

- Width: 14 pixels

- Height: 10 pixels

KEYBOARD

- Width: 16 pixels

- Height: 15 pixels

DIALOG

- Width for back/enter/edit: 20 pixels

- Height for  back/enter/edit: 20 pixels

TOGGLE ICONS

- Width: 25 pixels

- Height: 41 pixels

STUDIOLOGIC ICON

- Width: 25 pixels

- Height: 41 pixels

Example of an image positioning

It has not been mentioned that X = 0 and Y = 0 always refer to the top-left corner of the display. For example, X = 160 and Y = 120 correspond to the pixel at the center of the display. This is because 320/2 = 160 and 240/2 = 120.

However, the image will not be centered since the X and Y pair refers to the positioning point but does not take into account the WIDTH and HEIGHT of the image itself. In fact, to perfectly center the image, you need to compensate for the X and Y values by subtracting half the WIDTH and half the HEIGHT.

e.g.

Image width = 20

Image height = 30

X = 160 - 10

Y = 120 - 15

PLOT LCD RECTANGLE

This documentation should provide the necessary information to implement the DRAW RECTANGLE function in your applications using the provided API documentation.

The SysEx message to draw a rectangle follows this format.

Byte Breakdown

- 0xF0: Start SysEx

- 0x00: Manufacturer ID byte 0

- 0x20: Manufacturer ID byte 1

- 0x1A: Manufacturer ID byte 2

- 0x0E: Product ID

- 0xID: Host ID (app identification code)

- 0x04: Item type (display LCD)

- 0x02: Function (2 = draw rectangle)

- 0xXX_MSB: MSB of the x-coordinate of the top left vertex of the rectangle

- 0xXX_LSB: LSB of the x-coordinate of the top left vertex of the rectangle

- 0xYY_MSB: MSB of the y-coordinate of the top left vertex of the rectangle

- 0xYY_LSB: LSB of the y-coordinate of the top left vertex of the rectangle

- 0xWW_MSB: MSB of the width of the rectangle

- 0xWW_LSB: LSB of the width of the rectangle

- 0xHH_MSB: MSB of the height of the rectangle

- 0xHH_LSB: LSB of the height of the rectangle

- 0xcc: Color SEE COLOUR LIST (i.e. list_colour enumerator)

- 0xF7: Stop SysEx

typedef enum

{

// Base Colours

COLOUR_BLACK = 0,

COLOUR_WHITE,

COLOUR_GRY,

COLOUR_GRY_MUTE,

// Zone Colours

COLOUR_ZONE1,

COLOUR_ZONE2,

COLOUR_ZONE3,

COLOUR_ZONE4,

// Audio Colours

COLOUR_AUDIO_1,

COLOUR_AUDIO_2,

COLOUR_AUDIO_3,

COLOUR_AUDIO_4

} list_colour;

Below is the list of colors corresponding to the enumerator mentioned above, in 24-bit RGB:

Base Colours

r: 0, g: 0, b: 0 // Black

r: 255, g: 255, b: 255 // White

r: 115, g: 113, b: 115 // Grey

Defaults SL Zone Colours

r: 255, g: 161, b: 57 // Orange

r: 255, g: 238, b: 74 // Yellow

r: 131, g: 222, b: 90 // Green

r: 106, g: 198, b: 255 // Blue

Other Colours

r: 148, g: 149, b: 255 // Light Slate Blue

r: 148, g: 255, b: 222 // Aquamarine

r: 255, g: 117, b: 123 // Salmon

r: 255, g: 137, b: 255 // Fuchsia Pink

Example 1: Drawing a black rectangle at position (x=50, y=50) with width 100 and height 80

Assume the following values for the example:

- Host ID = 0x01

- X-coordinate = 50 (0x00 0x32 in hex)

- Y-coordinate = 50 (0x00 0x32 in hex)

- Width = 100 (0x00 0x64 in hex)

- Height = 80 (0x00 0x50 in hex)

- Color = Black (0)

The SysEx message would be:

0xF0 0x00 0x20 0x1A 0x0E 0x01 0x04 0x02 0x00 0x32 0x00 0x32 0x00 0x64 0x00 0x50 0x00 0xF7

Notes

- The coordinates (x, y) define the top left vertex of the rectangle.

- The MSB and LSB of the coordinates, width, and height are separated to allow for higher precision and larger values.

- The color is specified using the following scheme: 0 = black, 1 = white, 2 = grey, 3 = orange, 4 = yellow, 5 = green, 6 = blue, etc. SEE list_colour enumerator

PLOT LCD TEXT

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x04 →  item type: display LCD

- 0x00 → function: 0 = write text

- 0xXX_MSB → MSB of the x-coordinate of the top left vertex of the text rectangle

- 0xXX_LSB → LSB of the x-coordinate of the top left vertex of the text rectangle

- 0xYY_MSB → MSB of the y-coordinate of the top left vertex of the text rectangle

- 0xYY_LSB → LSB of the y-coordinate of the top left vertex of the text rectangle

- 0xLL_MSB → MSB of the length of the writable area

- 0xLL_LSB → LSB of the length of the writable area (if LL_MSB = 0 and LL_LSB = 0, the length of the writable area is the minimum one that fits the text

- 0xAA →alignement: SEE text_indent enumerator

- 0xff→ font size: SEE text_font enumerator

- 0xbf → background/foreground color pair SEE list_colour enumerator

- 0xtt → 1-st string character

...→ ““

- 0xtt → n-th string character

- 0x00 → string terminator

- 0xF7 → stop sysex

typedef enum

{

TEXT_INDENT_LEFT = 0,

TEXT_INDENT_CENTER,

TEXT_INDENT_RIGHT

} text_indent;

typedef enum

{

TEXT_FONT_SMALL = 0,    //size 21

TEXT_FONT_MIDDLE,       //size 27

TEXT_FONT_BIG           //size 33

} text_font;

typedef enum

{

TEXT_COLOR_WHITE_ON_BLACK,

TEXT_COLOR_BLACK_ON_ZONE1,  // ORANGE

TEXT_COLOR_BLACK_ON_ZONE2,  // YELLOW

TEXT_COLOR_BLACK_ON_ZONE3,  // GREEN

TEXT_COLOR_BLACK_ON_ZONE4,  // BLUE

TEXT_COLOR_BLACK_ON_WHITE,

TEXT_COLOR_BLACK_ON_GREY,

TEXT_COLOR_ZONE1_ON_BLACK,

TEXT_COLOR_ZONE2_ON_BLACK,

TEXT_COLOR_ZONE3_ON_BLACK,

TEXT_COLOR_ZONE4_ON_BLACK,

TEXT_COLOR_GREY_ON_GREY,

TEXT_COLOR_GREY_ON_GREY_MUTE,

TEXT_COLOR_GREY_ON_BLACK,

TEXT_COLOR_WHITE_ON_GREY,

TEXT_COLOR_GREY_ON_ZONE1,

TEXT_COLOR_GREY_ON_ZONE2,

TEXT_COLOR_GREY_ON_ZONE3,

TEXT_COLOR_GREY_ON_ZONE4,

TEXT_COLOR_BLACK_ON_AUDIO1,  // Light Slate Blue

TEXT_COLOR_BLACK_ON_AUDIO2,  // Aquamarine

TEXT_COLOR_BLACK_ON_AUDIO3,  // Salmon

TEXT_COLOR_BLACK_ON_AUDIO4,  // Fuchsia Pink

TEXT_COLOR_AUDIO1_ON_BLACK,

TEXT_COLOR_AUDIO2_ON_BLACK,

TEXT_COLOR_AUDIO3_ON_BLACK,

TEXT_COLOR_AUDIO4_ON_BLACK,

TEXT_COLOR_WHITE_ON_RED

} text_colour;

Parameters Explanation

- X-coordinate: The position of the top-left vertex of the text rectangle along the X-axis.

- Y-coordinate: The position of the top-left vertex of the text rectangle along the Y-axis.

- Length of the writable area: Defines the WIDTH of the area where text can be written.

- Alignment: Specifies the alignment of the text within the writable area (left, center, right).

- Font Size: Determines the size of the text font.

- Background and foreground Color: Specifies the pair for background and text color of the text area, see the list_colour enumerator.

- String Characters: The text to be written on the display.

Example

Here's an example of how to format the Sysex message to draw text on the SL-MKII controller's display:

F0 00 20 1A 0E ID 04 00 XX_MSB XX_LSB YY_MSB YY_LSB LL_MSB LL_LSB AA FF BF 54 65 73 74 69 6E 67 20 31 32 33 00 F7

In this example:

- XX_MSB, XX_LSB, YY_MSB, YY_LSB represent the X and Y coordinates of the top-left vertex of the text rectangle.

- LL_MSB, LL_LSB indicate the length of the writable area.

- AA specifies the alignment of the text.

- FF determines the font size.

- BF denotes the background and foreground (text) colors.

- 54 65 73 74 69 6E 67 20 31 32 33 is the string "Testing 123" to be displayed.

- 00 is the string terminator.

- F7 marks the end of the Sysex message.

Notes

- Ensure that the provided coordinates and the length of the writable area fall within the display bounds to avoid any text clipping or overflow.

- Choose appropriate font sizes and colors to ensure clear visibility and readability of the text on the display.

- Remember to terminate the string with 0x00 to indicate the end of the text string in the Sysex message.

CLEAR LCD DISPLAY

There are no substantial differences between using the 'Clear Display' message and drawing a rectangle covering the entire screen.

This Sysex message clears the content displayed on the SL-MKII controller's LCD screen.

Sysex Message Format

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x05 →  item type: display LCD

- 0x01 → function: 1 = clear display

- 0xcc → colore: SEE list_colour enumerator

- 0xF7 → stop sysex

Parameters Explanation

- Send 0x05 to clear LCD display.

- Color: Specifies the color to be used for clearing the display.

Example

F0 00 20 1A 0E ID 05 01 CC F7

Notes

- This message will remove all content currently displayed on the SL-MKII controller's LCD screen.

- Ensure that the color specified matches the background color of the display to effectively clear the screen.

- Verify compatibility with the SL-MKII controller's firmware version to ensure proper execution of the clear display function.

LED and Encoders

Up to this point, we've focused on sending messages to write on the LCD. The following APIs allow access to the LEDs.

The SL-MKII features two types of LEDs:

- Those associated with buttons, referred to as LED_WHITE.

- Those associated with encoders, referred to as LED_RGB.

The distinction between the two lies in the fact that the LED_WHITE can only display the color white when turned on, while the LED_RGB can be colored and have a 'brightness' parameter that defines their luminosity.

LED_WHITE

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x02 →  item type: led

- 0xnn →  led number: SEE list_led enumerator

- 0xss→  led state: 0 = spento, 1 = acceso

- 0xF7 → stop sysex

typedef enum

{

SELECT_ZOOM_1_LED,

SELECT_ZOOM_2_LED,

SELECT_ZOOM_3_LED,

SELECT_ZOOM_4_LED,

MODE_LED,

GLOBAL_LED,

DAW_LED,

NOT_USED_EX_SPLIT_LED,

NOT_USED_EX_EDIT_LED,

NOT_USED_EX_GLOBAL_LED,

APPLY_LED,

CANCEL_LED,

HOME_LED,

LED_NUM

} list_led;

LED_RGB

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x05 →  item type: led RGB

- 0xnn →  led number: SEE list_led_rgb enumerator

- 0xcc → led color: SEE list_led_rgb_colour enumerator

- 0xii →  led brightness: range 0-127 with 0 = min brightness (off) e 127= max brightness

- 0xF7 → stop sysex

typedef enum

{

LED_RGB_1 = 0,

LED_RGB_2,

LED_RGB_3,

LED_RGB_4,

LED_RGB_NUM

} list_led_rgb;

typedef enum

{

LED_RGB_COLOUR_BLACK,

LED_RGB_COLOUR_WHITE,

LED_RGB_COLOUR_ZONE1,

LED_RGB_COLOUR_ZONE2,

LED_RGB_COLOUR_ZONE3,

LED_RGB_COLOUR_ZONE4,

LED_RGB_COLOUR_AUDIO1,

LED_RGB_COLOUR_AUDIO2,

LED_RGB_COLOUR_AUDIO3,

LED_RGB_COLOUR_AUDIO4,

LED_RGB_COLOUR_NUM

} list_led_rgb_colour;

ENCODER

Encoders can only send messages; the reception of a message from an encoder must be handled by the recipient.

It is important to note that encoders do not send absolute values but rather send relative values. When turned clockwise, they send a positive 'tick'; conversely, when turned counterclockwise, they send a negative 'tick'.

The speed at which the encoder is rotated will produce a larger gap in values, for example, +2, +3, or -2, -3.

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x03 →  item type: encoder

- 0xnn →  encoder number: SEE list_encoder enumerator

- 0xtt (RECEIVED) → tick number (you must add +64 to the value because the available range is 0-127 but the tick number could be also negative. For example: 65 = +1 tick, 64 = -1 tick)

- 0xF7 → stop sysex

typedef enum

{

ENCODER_1 = 0,

ENCODER_2,

ENCODER_3,

ENCODER_4,

JOYSTICK_ENCODER,

ENCODER_NUM

} list_encoder;

Example of Message Reception and Value Increment:

In this example:

- relativeTick is the value received from the encoder. It is subtracted by 64 to handle the range of values correctly.

- The value of myStruct is incremented (or decremented if negative) by the relativeTick.

// Receive the relative tick value from the encoder

int relativeTick = item_val - 64;

// Increment the value in the structure myStruct

myStruct->value += relativeTick;

BUTTON

Like the Encoders, Buttons can only send messages,.

- 0xF0 →  start sysex

- 0x00 →  MANUFACTURER_ID byte 0

- 0x20 →  MANUFACTURER_ID byte 1

- 0x1A →  MANUFACTURER_ID byte 2

- 0x0E →  PRODUCT_ID

- 0xID → HOST ID (app identification code)

- 0x01 →  item type: button

- 0xnn →  button number: SEE list_button enumerator

- 0xpp (RECEIVED) → pression type -> 0 = no pression, 1 = short pression, 2 = long pression

- 0xF7 → stop sysex

typedef enum

{

ENCODER_BUTTON_1 = 0,

ENCODER_BUTTON_2,

ENCODER_BUTTON_3,

ENCODER_BUTTON_4,

SELECT_ZOOM_BUTTON_1,

SELECT_ZOOM_BUTTON_2,

SELECT_ZOOM_BUTTON_3,

SELECT_ZOOM_BUTTON_4,

MODE_BUTTON,

GLOBAL_BUTTON,

DAW_BUTTON,

NOT_USED_BUTTON_1,

NOT_USED_BUTTON_2,

NOT_USED_BUTTON_3,

APPLY_BUTTON,

CANCEL_BUTTON,

HOME_BUTTON,

JOYSTICK_BUTTON_UP,

JOYSTICK_BUTTON_LEFT,

JOYSTICK_BUTTON_DOWN,

JOYSTICK_BUTTON_RIGHT,

JOYSTICK_BUTTON_MAIN,

BUTTON_NUM

} list_button;

Note:

It is important to note that the same logic is applied to the push encoders, as described above. In fact, encoders also function as buttons.

Smooth and efficient MIDI communication

The connection between the SL-MKII and the device (App) is established via MIDI, making it essential to enable both reception and transmission of MIDI messages. To prevent congestion from excessive messages, the 'pulling' technique is used for both reading and writing.

This implementation ensures a smooth and efficient MIDI communication between the SL-MKII and the device, leveraging the 'pulling' technique to manage message flow effectively.

In essence, there are two timers, one for input and the other for output, which control the data flow at millisecond intervals.

std::vector<MidiMessage> queueIn;

std::vector<MidiMessage> queueOut;

void handleIncomingMidiMessage(MidiInput* source,

const MidiMessage& message)

{

queueIn.push_back(message);

}

void sendMidiMessage(const MidiMessage& sysMsg)

{

queueOut.push_back(sysMsg);

}

void timerCallback(int timerID)

{

switch (timerID)

{

case TIMER_MIDI_OUT:

if (queueOut.size() > 0)

{

MidiMessage sysMsg = queueOut.front();

queueOut.erase(queueOut.begin());

mOut->sendMessageNow(sysMsg);

}

break;

case TIMER_MIDI_IN:

if (queueIn.size() > 0)

{

MidiMessage sysMsg = queueIn.front();

queueIn.erase(queueIn.begin());

parseMidiInput(sysMsg);

}

break;

}

}

Code Explanation:

- Queues for MIDI Messages:

  - std::vector<MidiMessage> queueIn; - This queue stores incoming MIDI messages.

  - std::vector<MidiMessage> queueOut; - This queue stores outgoing MIDI messages.

- Handling Incoming MIDI Messages:

  - void handleIncomingMidiMessage(MidiInput* source, const MidiMessage& message) - This callback function is triggered by the system when a MIDI message is received. The message is added to queueIn.

- Sending MIDI Messages:

  - void sendMidiMessage(const MidiMessage& sysMsg) - This function adds outgoing MIDI messages to queueOut.

- Timer Callback Function:

  - void timerCallback(int timerID) - This function is called by a timer to process MIDI messages. There are two cases:

    - TIMER_MIDI_OUT: Checks if there are any messages in queueOut. If there are, it sends the first message and removes it from the queue.

    - TIMER_MIDI_IN: Checks if there are any messages in queueIn. If there are, it processes the first message and removes it from the queue.

Additional Notes:

- Timer Intervals: The timers operate at millisecond intervals, ensuring efficient data processing without overwhelming the system.

- Callback Function: handleIncomingMidiMessage is a callback function triggered by the system. MidiInput is  the input source, and MidiMessage is the MIDI message received.

- JUCE Library: Refer to the JUCE documentation for more details on handling MIDI messages and using the JUCE framework for audio and MIDI development.

MIDI Send

La funzione sendMessageNow() è membro della classe MidiOutput, il puntatore mOut è ottenuto dall’ AudioDeviceManager.

MidiOutput* mOut = audioDeviceManager->getDefaultMidiOutput();

if (mOut) mOut->sendMessageNow(sysMsg);

MIDI Receive

The following section demonstrates how received MIDI packets are interpreted. It's essential to analyze the MIDI messages contained in queueIn. Each MIDI message can represent different actions, such as button pressed, encoder changes, etc.

const uint8_t FATAR_SYSEX_ID[3] { 0x00, 0x20, 0x1A };

const uint8_t PRODUCT_ID = 0x0E;

const uint8_t HOST_ID = 0x01;

void parseMidiInput(const MidiMessage& message)

{

// MIDI input from SL-MKII came from "SL-MKII Porta 3"

if (message.isSysEx()) {

if (message.getSysExDataSize() < 7) return;

// Parse message according to sysex data

// [0], [1], [2] is the FATAR_SYSEX_ID

// [3] is the PRODUCT_ID

uint8_t fatar1 = message.getSysExData()[0];

uint8_t fatar2 = message.getSysExData()[1];

uint8_t fatar3 = message.getSysExData()[2];

uint8_t prodId = message.getSysExData()[3];

uint8_t hostId = message.getSysExData()[4];

if (fatar1 != FATAR_SYSEX_ID[0] || fatar2 != FATAR_SYSEX_ID[1]

|| fatar3 != FATAR_SYSEX_ID[2] || prodId != PRODUCT_ID

|| hostId != HOST_ID) return;

uint8_t item_id = message.getSysExData()[5];

uint8_t item_num = message.getSysExData()[6];

uint8_t item_val = message.getSysExData()[7];

switch (item_id)

{

case ITEM_SYSTEM_SYSTEM: {

switch (item_num) {

case SYSTEM_LOGIN_REQUEST:

DBG("login request");

break;

case SYSTEM_LOGIN_CONFIRM:

DBG("login confirmation");

break;

case SYSTEM_LOGOUT_REQUEST:

DBG("logout request");

break;

case SYSTEM_LOGOUT_CONFIRM:

DBG("logout confirmation");

break;

case SYSTEM_STANDBY:

DBG("standby notification");

break;

case SYSTEM_RESTART:

DBG("restart notification");

break;

}

This example illustrates how to read the values from various controls on the SL-MKII. This logic is applicable to all other control elements as well.

const uint8_t FATAR_SYSEX_ID[3] { 0x00, 0x20, 0x1A };

const uint8_t PRODUCT_ID = 0x0E;

const uint8_t HOST_ID = 0x01;

void parseMidiInput(const MidiMessage& message)

{

// MIDI input from SL-MKII came from "SL-MKII Porta 3"

if (message.isSysEx()) {

if (message.getSysExDataSize() < 7) return;

// Parse message according to sysex data

// [0], [1], [2] is the FATAR_SYSEX_ID

// [3] is the PRODUCT_ID

uint8_t fatar1 = message.getSysExData()[0];

uint8_t fatar2 = message.getSysExData()[1];

uint8_t fatar3 = message.getSysExData()[2];

uint8_t prodId = message.getSysExData()[3];

uint8_t hostId = message.getSysExData()[4];

if (fatar1 != FATAR_SYSEX_ID[0] || fatar2 != FATAR_SYSEX_ID[1]

|| fatar3 != FATAR_SYSEX_ID[2] || prodId != PRODUCT_ID

|| hostId != HOST_ID) return;

uint8_t item_id = message.getSysExData()[5];

uint8_t item_num = message.getSysExData()[6];

uint8_t item_val = message.getSysExData()[7];

switch (item_id)

{

case ITEM_BUTTON:

{

if (item_num == JOYSTICK_BUTTON_MAIN)

{

// JOYSTICK PUSH

}

else if (item_num == APPLY_BUTTON)

{

if (item_val == 1)

{

// APPLY_BUTTON PUSH

}

else if (item_val == 2)

{

// APPLY_BUTTON LONG PUSH

}

}

Reference Section

STUDIOLOGIC Official Website

Studiologic: Official website for Studiologic products including manuals, software updates, and support.

JUCE Official Website

JUCE: Official website for JUCE, a widely used C++ framework for developing cross-platform audio applications.

MIDI Manufacturers Association (MMA)

MIDI.org: The official site of the MIDI Manufacturers Association with resources, specifications, and news about MIDI technology.

Additional Resources

Audio Developer Conference (ADC)

Audio Developer Conference: Annual conference focused on audio development, often featuring JUCE-related content and talks.

GitHub Repositories

JUCE on GitHub: Official JUCE repository with source code and contributions from the community.

MIDI Projects on GitHub: Explore various MIDI-related projects on GitHub.

Books and Guides

Designing Audio Effect Plug-Ins in C++: A book by Will Pirkle that provides an in-depth look at audio effect plugin design using C++ and frameworks like JUCE.

MIDI Power!: The Comprehensive Guide: A comprehensive guide to understanding and using MIDI technology.

This reference section includes essential links for getting started and diving deeper into Studiologic products, JUCE framework, and MIDI technology.